UDC 629.73:351.814.2 (045)

V.P. Kharchenko, prof.1

O.M. Alexeiev, PhD2

K.M. Tapia, PhD student.3

COLLISION PROBABILITY OF AIRCRAFT FLYING IN PARALLEL TRACKS

1, 2, 3 National Aviation University

1Е-mail: kharch@nau.edu.ua

2Е-mail: oalexeiev@yahoo.com

3Е-mail: tapiae@mail.ru

Concepts of Air Traffic Management are focused on the geometry of airways. Today’s navigation facilities are able to increase airspace capacity due to more accurate path keeping and providing reduced position error. It will increase capacity of airspace but decrease safety level. According to this collision probability  between two aircraft flying on opposite tracks with reduced lateral separation is considered and compared with level of safety.

Концепции управления воздушным движением сосредоточены на геометрии воздушных трасс. Сегодня навигационные средства в состоянии увеличить пропускную способность воздушного пространства за счет более точного выдерживания траектории и предоставлении уменьшенной ошибки местоположения. Это позволит увеличить пропускную способность  воздушного пространства, но снизит уровень безопасности. В соответствии с этим рассмотрена вероятность столкновения двух воздушных судов на параллельных воздушных трассах при сокращенном боковом эшелонировании, а также  ее соответствие уровню безопасности.

Keywords: aircraft, collision probability, lateral separation, position error, safety.

Introduction

The commercial aviation industry is a significant component of the global economy. More than 4.5 percent of the world economic output is associated with the air transport component of civil aviation (International Civil Aviation Organization (ICAO), 1998).

The busiest airspace is congested for significant periods of time due to a combination of traffic volume, weather and other airspace restrictions. The problem is to achieve the highest levels of efficiency and safety. Airspace management concepts (AMC) attempt to mitigate required traffic flow management and allow more user preference and traffic flexibility. According to Seven-Year Flight Forecast of Eurocontrol by 2018 anticipates 11.1 million instrument flight rules (IFR) movements in Europe. From 2014 it will grow in averages 1.9% per year for the whole 2012-18 period. Flight delays rise with traffic growth. The cause for this delay is commonly referred to as a ‘lack of capacity’.  Capacity of the airspace is constrained by many factors including number of runways, aircraft separation requirements, and the Air Traffic Controller (ATC) workload limitations.

ATM design

Safety/Capacity assessment

Figure 1. Dependence between airspace structure, capacity and safety.

Any increase in capacity must be accompanied by a demonstration that such an increase will be safe. As capacity of airspace increases, the relative safety must decrease to achieve the same absolute accident rate over time.

Literature overview

The existing Air Traffic Service (ATS) route structure often involves mileage penalties, compared to the most economic routes, which may be great circle routes but which also take into account wind, temperature and other factors such as weight of the aircraft, charges and safety.

According to navigation facilities one of the possibilities to increase airspace capacity is to reduce separation minima [6]

Aircraft separation standards use space and time divisions to prevent conflict and confrontation. On one side Separation minima reductions will increase capacity of airspace, but on the other side growing air transport demand will provide rising risk of conflicts and collisions [8].

Safety depends of  the density and complexity of air traffic and overall air traffic states (for winter or summer, weather patterns etc.) [3].

Concepts of «capacity», «safety» and «technology» are focused on the geometry of airways:

- capacity increases for smaller separation  L;

- navigation and flight technology should provide a reduced root mean square (r.m.s.) position error ;

- combination of  L and   should be such that the probability of collision does not exceed ICAO  Target Level of Safety (TLS) of 5*10-9  per hour [1,4,6].

The actual motion of the aircraft is affected by uncertainty, due mainly to wind, but also to errors in tracking, navigation, and control. The deferent sources of uncertainty affecting the aircraft motion generally cause deviations from the nominal flight path.

Thus the goal of this paper is to determine the collision probability between two aircraft flying on opposite tracks with reduced lateral separation, which compares with the ICAO TLS.

Collision probability of aircraft flying in parallel tracks.

Large aircraft flight deviations are rare events and the r.m.s. position error is an indirect safety metric because it specifies an upper bound for the probability of collision [1]. The r.m.s. position error is easy to implement as a safety metric: it just requires monitoring of the deviation between the actual trajectory and the intended trajectory.

Fk(x;σ)=Aexp(−axk),

where k is the weight. Constant a is determined by the condition of unit total probability:

A=ka1k2Γ1k

The r.m.s. position error σ or variance σ2

k=1,  a=2σ , A=1σ2.

It leads to:

F1x;σ=1σ2exp(−2xσ)

For statistically independent aircraft deviations at position x:

Pk(x;L,σ1,σ2)=Fk(x;σ1)Fk(L−x;σ2)

Consider two aircraft flying at a constant lateral separation L in parallel flight paths. In this collision scenario, the aircraft can at all times drift into positions less than a minimum separation distance. A coincidence will occur if position errors are x (aircraft 1) and L-x (aircraft 2).

Figure 2. Lateral separation between aircraft.

An integral over all positions along the line joining the two aircraft:

Qk(L;σ1,σ2)=−∞+∞Pk(x;L,σ1,σ2)dx=−∞+∞Fk(x;σ1)Fk(L−x;σ2)dx

The collision probability has the dimensions of inverse length. The ICAO TLS of 5*10-9 per hour can be converted for a maximum airspeed V0 = 625 kt to an ATLS.

The safety criterion is Q0=5×10−9. Upper bound for V0 625kt, is  Q≤Q0=Q0V0 which apply the ATLS 8*10-12   nm-1 .

Two aircraft with dissimilar r.m.s. position errors follow the next probability distribution:

Q0≥Q1(L;σ1,σ2)=12σ1σ2−∞+∞exp−2xσ1+L−xσ2dx

The appearance of modulus in the argument of the exponential requires that the range of integration, be split in three parts.

1. The flight paths between of the two aircraft at0≤x≤L.

2σ1σ2Q11=0Lexp−2xσ1+L−xσ2dx=exp−2σ20Lexp−2x1σ1−1σ2dx

and involves an integration:

2σ1σ2Q11=exp−2Lσ2×1−exp−2L1σ1−1σ221σ1−1σ2−1

and simplifies to:

Q11=22σ2−σ1−1××exp−2Lσ2−exp−2Lσ1

.

2. The remaining contributions at a point   outside the path of second aircraft:

2σ1σ2Q12=L∞exp−2xσ1+x−Lσ2dx

leads to an integral:

2σ1σ2Q12=exp2Lσ2L∞exp−2x1σ1+1σ2dx=exp2Lσ221σ1+1σ2−1exp−2L1σ1+1σ2

which simplifies to:

Q12=22σ1+σ2−1exp−2Lσ1

3. The coincidence <x<0 outside the flight path of the first aircraft:

2σ1σ2Q13=−∞0exp2xσ1−L−xσ2dx=exp−2σ20∞exp−2x1σ1+1σ2dx

then

Q13=22σ2+σ1−1exp−2Lσ2

The sum of Q11,Q12,Q13 where:

Q1(L;σ1,σ2)=22σ2−σ1−1××exp−2Lσ2−exp−2Lσ1++22σ2+σ1−1exp−2Lσ1+exp−2Lσ2

for distribution:

Q1(L;σ1,σ2)=Q11+Q12+Q13≤Q0=8∗10−12nm

The preceding safety-separation criteria are applied to lateral separation in controlled airspace with standard and reduced separation.

The standard deviation for VOR/DME is taken as 0.3 NM. The  GNSS navigation is highly accurate in the lateral path keeping.

Let compare the collision probability for lateral separation in 5, 4, 3 nm in controlled airspace with the ATLS probability of collision per nautical mile in controlled airspace with  σ1,σ2≤0.3nm.

Figure 3. Probability of collision aircraft with dissimilar r.m.s. position errors.

The r.m.s. position error of two aircraft where σ1=σ2≡σ

1. The flight paths between two aircraft when 0≤x≤L:

Q11=2σ2−10Lexp−2Lσdx=L2σ2exp−2Lσ

2. The remaining contributions at a point   outside the path of second aircraft:

Q12=2σ2−1exp−2LσL∞exp−22xσdx=42σ−1exp−2Lσ

3. The coincidence < x <0  outside the flight path of the first aircraft

Q13=2σ2−1exp−2Lσ−∞0exp−22xσdx=42σ−1exp−2Lσ

The sum specifies:

Q1(L;σ)=exp(−2Lσ)(2σ)−1Lσ+12

safety criterion:

Q1(L;σ)=Q11+Q12+Q13≤Q0=8∗10−12nm−1

Figure 4. Probability of collision aircraft with equal r.m.s. position errors.

Probability of collision for L<4 nm exceed the ICAO TLS both dissimilar and equal r.m.s. position errors. It is mean that navigation aids should provide r.m.s. position error σ<0.3 nm of flight path deviations to reduction lateral separation.

Conclusions

Safety standards of ICAO require probability of collision of  the odder 5*10-9  per hour. In the case of aircraft flying in opposite directions in parallel tracks, if the r.m.s. position error will be less than one tenth of the minimum separation distance, the probability of collision will be less then ICAO TLS.

The standard deviation for VOR/DME won't allow maintaining safety lateral separation when it will be less 5 nm. The GNSS navigation is highly accurate in the lateral path keeping.  Thus, implementation of more accurate navigation procedures will allow to reduce separation minima and increase capacity of airspace.

References

- Campos L., Marques J. Collision Probabilities, Aircraft Separation and Airways Safety / Prof. Max Mulder //Aeronautics and Astronautics—2011—610 p.

- Elefante S., Statistical and probabilistic approach for improving efficiency in Air Traffic Flow Management//Dissertation — Faculty of Engineering at the University of Glasgow — 2001—241 p.

- Eurocontrol Metrics & Methodologies  Safety Metrics Technical Definitions CARE-INTEGRA Report to Eurocontrol, Brussels, 2000, 19 p.

- Masao K., Dr. Masato F. Safety Assessment prior to 30NM Longitudinal Separation Minimum under ADS-C Environment/-Montreal, Canada: International Civil Aviation Organization, 2008, 5 p.

- Moek G., Smeltink J.W. Pre Implementation Collision Risk Assessment for RVSM in the Africa Indian Ocean Region/National Aerospace Laboratory — Air Traffic and Navigation Services- 2005—133 p.

- Netjasov F. Conflict risk assessment model for airspace strategic planning/Division of Airports and Air Traffic Safety.—Belgrade, Serbia, 2010—16p

- Netjasov F., Milan Janic A Review of the Research on Risk and Safety Modeling in Civil Aviation/Third International Conference on Research in Air Transportation — ,2008, 8p

- Thompson S.D, Terminal Area Separation Standards: Historical Development, Current Standards, and Processes for Change/ Project Report ATC-258. — Springfield, Virginia: Lincoln Laboratory of Massachusetts Institute of Technology, 1997. — 60 p.

- Yiyuan J. Z. A Systematic procedure for determining separation minima/ICAS—2008