(F 03.02 – 89)

- National Aviation University

Institute (Faculty) Scientific & Research Institute of Air Navigation

Department Aviation English

Field of Study 6.051103 Avionics

- Subject «Foreign Language»

Complex TEST

VARIANT № __1__

Electrical system

Airplanes are equipped with either a 14- or 28-volt direct-current electrical system. A basic airplane electrical system consists of the following components: Alternator/generator;     Battery; Master/battery switch; Alternator/generator switch; Bus bar, fuses, and circuit breakers;    Voltage regulator; Ammeter/loadmeter; associated electrical wiring.

Engine-driven alternators or generators supply electric current to the electrical system. They also maintain a sufficient electrical charge in the battery. Electrical energy stored in a battery provides a source of electrical power for starting the engine and a limited supply of electrical power for use in the event the alternator or generator fails.

Most direct current generators will not produce a sufficient amount of electrical current at low engine r.p.m. to operate the entire electrical system. Therefore, during operations at low engine r.p.m., the electrical needs must be drawn from the battery, which can quickly be depleted.

Alternators have several advantages over generators.

Alternators produce sufficient current to operate the entire electrical system, even at slower engine speeds, by producing alternating current, which is converted to direct current. The electrical output of an alternator is more constant throughout a wide range of engine speeds.

Some airplanes have receptacles to which an external ground power unit (GPU) may be connected to provide electrical energy for starting. These are very useful, especially during cold weather starting. Follow the manufacturer´s recommendations for engine starting using a GPU.

The electrical system is turned on or off with a master switch. Turning the master switch to the ON position provides electrical energy to all the electrical equipment circuits with the exception of the ignition system.

1. Read the text and answer the following questions:

1. What are the problems with the direct current alternators for the entire electrical system?

2. What are the sources of power for engine starting?

3. What equipment can be useful for starting during cold weather?

2. Find synonyms to these words and expressions in the text:

Permanent / adequate / preference / machinery / to keep, to save

3. Write down the essay (approximately 15 sentences). What are the advantages of using alternators over generators?

- Head of the Department

______________N. Pazuyra

“____”_________20___